July 9, 2010

The Fuel Procurement Department of the American Electric Power Service Corporation (“AEPSC”) is seeking bid proposals on behalf of one or more of its operating companies for the supply of coal to one or more power plants.  AEPSC may enter into one or more agreements, each for the supply of coal delivered by rail commencing as early as January 1, 2011.

Specifically sought are coals that: (1) have sulfur dioxide values (pounds SO2 per million Btu) meeting the requirements on the attached Coal Specification Table; and (2) meet the other quality specifications and supply parameters referenced on the attached Coal Specification Table.

The awarded purchase order(s) will be governed by the negotiated terms and conditions contained in the currently effective Master Coal Purchase and Sale Agreement (Master Agreement) between AEPSC and the successful bidder(s).  If the successful counterparty has not executed a Master Agreement with AEPSC, the imbedded object below represents the typical Terms and Conditions that will exemplify the awarded Fuel Purchase Order for a spot proposal of one year or less.

AEPSC is seeking the following proposals:

- That which provides for up to 3.0 million tons total spot coal beginning in January 1, 2011 for one year;

- That which provides for up to 3.0 million tons total spot coal beginning in January 1, 2012 for one year;

The proposal(s) should include a stated price or prices for the relevant terms of the proposal(s).  Those offering coal are invited to submit their offers on one or more of the proposals and may make multiple offers on any proposal.

A “Coal Proposal Form” and a “Rail Transportation Facilities Information” sheet shall comprise a “Coal Proposal Package”.  Copies of these forms are contained herein.  In order to ensure that proposals are responsive to AEPSC’s needs and provide a uniform basis for evaluation, those providing offers are requested to complete a Coal Proposal Package for each submitted offer.  Typical coal analyses, including the Proximate, Ultimate, Trace Element and Mineral Ash Analyses; Grindability Index; and Ash Fusion Temperatures, should be submitted with the attached Coal Proposal Package.

The Coal Proposal Package(s), from a person having proper authority to legally obligate the bidder, along with any additional supporting material, must be received no later than 5:00 p.m., Monday, July 19, 2010.

Proposals for spot coal are to be kept open for AEPSC’s acceptance until 5:00 p.m., Friday, July 23, 2010.

All proposals are to be submitted by e-mail to coalrfp@aep.com, or by fax to (614) 583-1619 (Attention: Chuck West), or delivered in a sealed envelope marked "AEPSC WESTERN RFP - PROPOSAL" and addressed as follows:

AEPSC Western RFP - Proposal

ATTN: Chuck West

American Electric Power Service Corp.

155 W. Nationwide Blvd.

Suite 500

Columbus  OH  43215

Proposals will be evaluated based upon the following factors:

- delivered cost (cents per million Btu),

- coal quality with appropriate price adjustments for guaranteed percent ash and pounds SO2 per million Btu

- reliability of supply and transportation flexibility

- terms and conditions

- previous experience with bidder and cooperation of bidding company in dealings with AEPSC

AEPSC reserves the right to modify or withdraw this invitation, to reject any or all proposals, and to terminate any subsequent negotiations at any time.  AEPSC also reserves the right to accept one or more proposal(s) for a portion or for all of the quantities of coal it seeks to acquire with this invitation.

AEPSC will consider every submitted offer on a confidential basis.

The bidder shall be responsible for the completeness and accuracy of all information contained in and used in preparation of its proposal(s) and for having supplied all necessary information.

If you do not wish to submit a proposal in response to this bid solicitation, please complete and return the attached, Decline to Offer form or send an e-mail to coalrfp@aep.com on or before Monday July 19, 2010.  Failure to return this form or an e-mail indicating same, in lieu of submitting a proposal, may result in your removal from our bid list.

Should further information be required, please contact Chuck West, 614.583.6112 or Tina Sefcik, 614.583.6028.

Very truly yours,

Stephen M Debord

Vice President, Fuel Procurement

AMERICAN ELECTRIC POWER SERVICE CORPORATION

### COAL SPECIFICATION TABLE

|  | Weighted Average "As-Received" Basis |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Specification “A” | Contracted | Half-Month (A)* |  | Applicable Lot (B)* |  |
|  |  | Suspension Limit |  | Rejection Limit |  |
|  |  | Minimum | Maximum | Minimum | Maximum |
| Calorific Value (Btu/lb.) | 8,800 | N/A | N/A | 8,600 | N/A |
| Moisture (%) | 27.00 | N/A | N/A | N/A | 30.00 |
| Ash (%) | 5.50 | N/A | N/A | N/A | 6.50 |
| Sulfur Dioxide (lbs. SO2/MMBtu) (C)* | 0.80 | N/A | N/A | N/A | 1.20 |

|  | Weighted Average "As-Received" Basis |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Specification “B” | Contracted | Half-Month (A)* |  | Applicable Lot (B)* |  |
|  |  | Suspension Limit |  | Rejection Limit |  |
|  |  | Minimum | Maximum | Minimum | Maximum |
| Calorific Value (Btu/lb.) | 8,550 | N/A | N/A | 8,350 | N/A |
| Moisture (%) | 28.50 | N/A | N/A | N/A | 31.00 |
| Ash (%) | 5.50 | N/A | N/A | N/A | 5.50 |
| Sulfur Dioxide (lbs. SO2/MMBtu) (C)* | 0.80 | N/A | N/A | N/A | 1.00 |

|  | Weighted Average "As-Received" Basis |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Specification “C” | Contracted | Monthly (A)* |  | Applicable Lot (B)* |  |
|  |  | Suspension Limit |  | Rejection Limit |  |
|  |  | Minimum | Maximum | Minimum | Maximum |
| Calorific Value (Btu/lb.) | 8,400 | N/A | N/A | 8,250 | N/A |
| Moisture (%) | 31.00 | N/A | N/A | N/A | 32.00 |
| Ash (%) | 5.50 | N/A | N/A | N/A | 6.50 |
| Sulfur Dioxide (lbs. SO2/MMBtu) (C)* | 0.80 | N/A | N/A | N/A | 1.20 |

N/A = Not Applicable

* Definitions:

(A) =  the half-monthly or monthly, as applicable, weighted average analysis result.

(B) = the analysis result of the sample (or composite of samples, if more than one) representing each unit train of coal.

(C) = For the purpose of determine the pounds of sulfur dioxide per million Btu, the figures shall be rounded to the nearest one hundredth.  For example, 0.954 pounds SO2/MMBtu shall mean 0.95 pounds SO2/MMBtu while 0.955 pounds SO2/MMBtu shall mean 0.96 pounds SO2/MMBtu and shall be deemed, for example, not to have met a 0.95 pounds SO2/MMBtu specification.

Please bid coal based on the specification that most closely meets the quality of the coal being offered.

Spot Proposal

SPOT COAL PROPOSAL FORM

DATE:

To:	 AEPSC Western RFP - Proposal

ATTN: Chuck West

American Electric Power Service Corp.

155 W. Nationwide Blvd.

Suite 500

Columbus  OH  43215

Details concerning the supply of coal being offered herein to the American Electric Power Service Corp. are provided below:

1.  Seller(s):  (If Seller is a Sales Agent, include full names, addresses, and telephone numbers of Sales Agent and Producer and attach Producer's signed authorization for Sales Agent to “exclusively” make an offer on behalf of Producer.)

(a)  Seller(s):	(b)  Producer(s):

_________________________________	________________________________

_________________________________	________________________________

_________________________________	________________________________

_________________________________	________________________________

2.  Name(s) and location(s) of mine(s):  (Include name of mine operating company).

__________________________________________________________________________________

__________________________________________________________________________________

__________________________________________________________________________________

3.  Quantity offered (tons per month):  ___________________________________________________

4.  Term:  Up to one calendar year; actual length of term shall be designated by AEPSC in the event of its acceptance.  Seller is to specify the earliest month coal is available:

__________________________________________________________________________________

5.  Shipping Point(s):

Rail Deliveries

Origin Station:

No. of railcars/shipment* (>125):

Maximum Loadout Time:   24 hrs.        4 hrs. 

6.  Price per ton (as applicable):	FOB Railcar:  $

Spot Proposal

SPOT COAL PROPOSAL FORM

7.	The quality of spot coal being offered herein is intended to meet one of the following specifications as stated on the Coal Specification Table:  	 Product A	 Product B	 Product C

8.	Guaranteed coal specifications “As-Received”:

Contracted		  Applicable Lot** 								 Half-Month  		    Minimum/Maximum***

a)  Calorific Value (Btu per pound)    	___________ 		_____________   Minimum

b)  Total Moisture (%)                  	___________ 	 	_____________   Maximum

c)  *Ash (%)                             		___________ 		_____________   Maximum

d)  Volatile Matter (%)                 	___________ 		_____________   Minimum

e)  Grindability (Hardgrove Index)       	___________ 		_____________   Minimum

f)  Ash Fusion Temperature (H = 1/2 W), oF

in Reducing Atmosphere	         	___________ 		_____________   Minimum

g)  Sulfur (%)                           		___________  		_____________   Maximum

h)  *SO2 (lbs./MMBtu)              		___________		_____________   Maximum

i)  Sodium Oxide (%)			___________		_____________   Maximum

j)  Coal Top Size    	                 				_____________

*	Offer evaluations will include appropriate price adjustments for offers which provide for lower Contracted percent ash and, as applicable, lower Contracted Half-Month SO2.

**	Applicable Lot refers to each unit trainload of coal.

9.	Type of Mine(s):  Surface _______%   Underground  _______%    Other  _______%

10.	 Mine District:  ___________________________________________________________________

11.	 Name of Seam(s):  ________________________________________________________________

12.  Preparation:   Crushed, run-of-mine   ______%           Washed   _______%	Synfuel  ________%

13.  Comments:  ____________________________________________________________________

____________________________________________________________________________________________________________________________________________________________________

THIS IS A FIRM OFFER TO SUPPLY FUEL UNDER ALL TERMS AND CONDITIONS INCLUDED IN THIS PROPOSAL PACKAGE, AND SHALL BE HELD OPEN BY SELLER FOR AEPSC’S ACCEPTANCE THROUGH 5:00 P.M., FRIDAY, JULY 23, 2010 .  UNTIL AEPSC’S ACCEPTANCE HEREOF, WHICH SHALL BE AT ITS SOLE DISCRETION, THIS PROPOSAL SHALL NOT CONSTITUTE A VALID AND ENFORCEABLE AGREEMENT, BUT UPON AEPSC'S ACCEPTANCE, SUCH PROPOSAL SHALL CONSTITUTE A VALID AND ENFORCEABLE AGREEMENT BETWEEN AEPSC AND SELLER.

Proposal submitted by:

(Signature)

(Title)

(Company)

AMERICAN ELECTRIC POWER SERVICE CORPORATION

AEPSC WESTERN RFP - PROPOSAL

RAIL TRANSPORTATION FACILITIES INFORMATION

The following information is required for each rail coal origin that is being bid:

Shipping Point_______________; Railroad(s) _______________; OPSL No. _______________

1.  What is the minimum and maximum unit train size that can be loaded?

2.  How fast can the indicated unit train be loaded?

3.  How precisely can the tonnage being loaded in each car be controlled?  What car loading precision would be guaranteed?

4.  Can the loading of the Unit Train start at any time and on any day upon the arrival of the cars?

5.  Is the assistance of the Railroad's locomotive and crew required to load the train?

6.  Are there any restrictions that would prevent your ability to load Buyer's 4,000 cubic feet AEPX hopper cars?

7.  How fast, after loading, can an advanced shipping manifest be FAXED to a location?

8.  Is equipment available for applying freeze-proofing and/or side release agents?

If there are any questions regarding this information being sought, please contact Jeff Dial at (614) 583-6972.

Signature

Title

Company

"DECLINE TO OFFER" FORM

TO AEP:  THIS FORM WILL SERVE AS OUR NOTICE TO YOU THAT WE ARE NOT SUBMITTING A PROPOSAL IN RESPONSE TO THIS CURRENT SOLICITATION.

PLEASE (CHECK ONE):   	 	RETAIN US ON THE LIST FOR FUTURE SOLICITATIONS

 	TAKE US OFF THE LIST FOR FUTURE SOLICITATIONS

COMPANY 	 	SIGNATURE

ADDRESS  	   	DATE

PHONE

RETURN BY:  July 19, 2010

  First Fold  

---------------------------------------------------------------------------------------------------------------------------------------------------------------Tape Here --

  Second Fold  

-------------------------------------------------------------------------------------------------------------------------------------------------

Place Postage Here

AEPSC WESTERN RFP

ATTN: CHUCK WEST

AMERICAN ELECTRIC POWER SERVICE CORP

155 W. NATIONWIDE BLVD.

SUITE 500

COLUMBUS OH  43215